ThorlabsLTS 150M User Manual

DESY

- Introduction:

Install and run ThorlabsLTS150M device on new computer.

- Procedure:

- Steps to install kinesis software:

- Click on the link shown below:

https://www.thorlabs.com/software_pages/viewsoftwarepage.cfm?code=Motion_Control

There are 3 options available

First 2 available for 32 and 64 bit operating system. Third option does not support labview

(32 bit) software.

- Click on download button.

- Download setup file and install it.

- Setup will extract files.

- Click on Next

- Accept license agreement

- Write information

- Select Complete option

- Click on install

- Click on Finish

- Create New Device For ThorlabsMDT Devicetype:

- Open Master control system. There is “DB Editor” button available. Click on that.

- Database Editor will open.

- Then go to Device ops > Device > New Devicetype.

- Then Enter password window will open. Enter password ”controlsystem”.

- Enter Name “ThrolabsLTS” and Relative Path “ThorlabsLTS\ThorlabLTS.lvclass” and click on create/Modify Devicetype.

- Click on Exit Main Program.

- Create New ThorlabsLTS150M device:

- Go to DB Editor.

- Click on Device Ops > Device > Device (add).

- Fill all details as shown below.

- Launch New ThorlabsLTS150M device:

- Go to devices tab in master control.

- Select ThrolabsLTS150M Device from list.

- Click on red button under Device tab.

- Then Device driver window of ThorlabsLTS150M will launch.

- When device launched successfully red button on master control will turn into green color.

- Then click on Stop button under diagnostic tab in device.

- Device will close.

- Modify Devicetype Variable for ThorlabsMDT693B device:

- Click on “DB editor” button again.

- Go to Device Ops > DeviceType variable Ops > Devicetype Variable (Add/Edit/Delete).

- New Windows will open. Select “ThrolabsLTS”Devicetype from that list of Devicetype.

- Then Click on variable.

- Select Simulation Variable from list.

- Click on Default value choices and select off.

- Click on Set Devicetype Variable Button.

- Then Simulation mode for device ThorlabsLTS150M will turn off.

- Select Serial Number variable and set its default value 83841487.

- Click on Exit Main Editor.

- Again Launch Device as per steps shown under “Launch New ThorlabsLTS150M device”.

- In Master Control, Select Device ThorlabsLTS150M device in Device selector.

- User can change devicetype variable and value.